Cisco IPsec VPN Lab (Packet Tracer)


This project demonstrates how to configure a site-to-site IPsec VPN between two Cisco routers using Packet Tracer. The lab includes:

- IPsec configuration with pre-shared keys

- ACL for defining interesting traffic

- Static routing for LAN-to-LAN connectivity

- Verification and troubleshooting commands

Let’s break down each command in your configuration and explain why it’s needed and what role it plays in building an IPsec VPN:

✅ 1. access-list 100 permit ip 192.168.1.0 0.0.0.255 192.168.2.0 0.0.0.255

- Purpose: Defines “interesting traffic” for IPsec.

- Why needed: IPsec does not encrypt all traffic by default. This ACL tells the router to encrypt packets between the two LAN subnets (192.168.1.0 and 192.168.2.0).

- Key point: If this ACL is missing or wrong, the tunnel will never come up.

✅ 2. crypto isakmp policy 10

- Purpose: Creates an ISAKMP (IKE Phase 1) policy.

- Why needed: Phase 1 establishes a secure channel for negotiating IPsec parameters.

- Options explained:

  - encryption aes 256 → Strong encryption for Phase 1.

  - authentication pre-share → Uses pre-shared key for authentication.

  - group 5 → Diffie-Hellman group for key exchange.

✅ 3. crypto isakmp key Ram@123 address 209.168.2.2

- Purpose: Defines the pre-shared key for the remote peer.

- Why needed: Both routers must have the same key for authentication in Phase 1.

- Key point: If the key or peer IP is wrong, Phase 1 fails.

✅ 4. crypto ipsec transform-set HQ->Br1 esp-aes 256 esp-sha-hmac

- Purpose: Defines the encryption and integrity algorithms for IPsec (Phase 2).

- Why needed: Specifies how data will be protected inside the tunnel.

- Options explained:

  - esp-aes 256 → Encrypts data with AES-256.

  - esp-sha-hmac → Provides integrity using SHA.

✅ 5. crypto map IPSEC-MAP 10 ipsec-isakmp

- Purpose: Binds all IPsec settings together and applies them to an interface.

- Why needed: Without a crypto map, the router won’t know which traffic to encrypt.

- Subcommands:

  - set peer 209.168.2.2 → Remote VPN peer IP.

  - set pfs group5 → Enables Perfect Forward Secrecy for extra security.

  - set security-association lifetime seconds 86400 → Tunnel lifetime (24 hrs).

  - set transform-set HQ->Br1 → Uses the defined transform set.

  - match address 100 → Links the ACL to the crypto map.

✅ 6. int gig 0/0

crypto map IPSEC-MAP

- Purpose: Applies the crypto map to the physical interface.

- Why needed: This is where encryption happens. If you forget this step, the tunnel never activates.

Summary of Why Each Is Needed

- ACL → Defines what to encrypt.

- ISAKMP policy & key → Secure Phase 1 negotiation.

- Transform set → Defines Phase 2 encryption/integrity.

- Crypto map → Combines everything and applies to interface.

- Interface binding → Activates IPsec on the WAN link.